Custom CAM operation steps

- Add pictures

Picture folder path: install \CAM

Image properties:

Resolution: 360 * 180

Bit depth: 24bit

Picture Format: BMP

The image file name corresponds to:

| menu | Image file name |
| --- | --- |
| CAM1 | cam10.bmp |
| CAM2 | cam11.bmp |
| CAM3 | cam12.bmp |
| CAM4 | cam13.bmp |
| CAM5 | cam14.bmp |
| CAM6 | cam15.bmp |
| CAM7 | cam16.bmp |
| CAM8 | cam17.bmp |
| CAM9 | cam18.bmp |
| CAM10 | cam19.bmp |

Example：

- Add parameters

Chinese Parameter Folder Path: Install\chs

English Parameter Folder Path: Install\eng

Parameter number range 1100-1499;

Macro address: # 2600- # 2999

Parameter index corresponding table:

| menu | Parameter index |
| --- | --- |
| CAM1 | m30 |
| CAM2 | m31 |
| CAM3 | m32 |
| CAM4 | m33 |
| CAM5 | m34 |
| CAM6 | m35 |
| CAM7 | m36 |
| CAM8 | m37 |
| CAM9 | m38 |
| CAM10 | m39 |

Example：

#1100 -p0 -a3  -t0 -s1"Parameter name 1" -s2" " -m30 -min=-9999.999 -max=99999.999

-s3"Description of parameter 1."

#1101 -p0 -a3  -t0 -s1"Parameter name 2" -s2" " -m30 -min=-9999.999 -max=99999.999

-s3"Description of parameter 2."

#1102 -p0 -a3  -t0 -s1"Parameter name 3" -s2" " -m30 -min=-9999.999 -max=99999.999

-s3"Description of parameter 3."

#1103 -p0 -a3  -t0 -s1"Parameter name 1" -s2" " -m31 -min=-9999.999 -max=99999.999

-s3"Description of parameter 1."

#1104 -p0 -a3  -t0 -s1"Parameter name 2" -s2" " -m31 -min=-9999.999 -max=99999.999

-s3"Description of parameter 1."

#1105 -p0 -a3  -t0 -s1"Parameter name 3" -s2" " -m31 -min=-9999.999 -max=99999.999

-s3"Description of parameter 3."

- Add corresponding processing code

File storage path: / local

Processing code correspondence:

| menu | Processing file corresponding table |
| --- | --- |
| CAM1 | cam10.nc |
| CAM2 | cam11.nc |
| CAM3 | cam12.nc |
| CAM4 | cam13.nc |
| CAM5 | cam14.nc |
| CAM6 | cam15.nc |
| CAM7 | cam16.nc |
| CAM8 | cam17.nc |
| CAM9 | cam18.nc |
| CAM10 | cam19.nc |

Example：

- Function Key Subject Number

Corresponding shortcut key ratios: 1399

- Operation

- The 210 parameter is set to 1399;

- Copy the corresponding parameters and processing files to the corresponding file directory,

- Press K1 button

- Select the corresponding CAM;

- Set parameters;

- Press the start button to process;