Internal Execution Principle of Mechanical Homing Sequence in M350

1：When performing full-axis mechanical homing using the F key on the system interface:

File: fndzero.nc
You can adjust the order of the following codes:

M98 P501 X2   // Z-axis

M98 P501 X0   // X-axis

M98 P501 X1   // Y-axis

M98 P501 X3   // 4th axis

M98 P501 X4   // 5th axis

2:For automatic mechanical homing on startup:

Files: sysstart.nc → slib-m.nc → M115
You can adjust the order of the following codes:

O10115

…

IF #1992 == 0 GOTO2   // The internal system parameter #127 is passed to #1992

M98 P501 X2   // Z-axis

M98 P501 X0   // X-axis

M98 P501 X1   // Y-axis

M98 P501 X3   // 4th axis

M98 P501 X4   // 5th axis

…  ……..